| A JSW ONE PRODUCT | STANDARD OPERATING PROCEDURE | Document ID:JODL/PB PT/01 |
| --- | --- | --- |
|  |  | Version: 1 |
|  | Incoming Coil Receiving | Date: 02/02/2026 |

Purpose:

To receive the incoming HR coils in the system.

Scope:

Each incoming coil

Teams and Responsibilities:

| Sl No | Team | Responsibility |
| --- | --- | --- |
| 1 | Contract Manufacturer | Joint Inspection of incoming coils and necessary communication with suppliers in case of any discrepancies |
| 2 | SGS QC Engineers | Joint Inspection of incoming coils, provide acceptance, handling rejection coils and report preparation |

- Receipt & Documentation Verification

- Verify PO: Cross-check grade, size, quantity of each supplied batch with Purchase Order (PO) details.

- Check MTC: Review Material Test Certificates for each batch for grade, composition and mechanical properties.

- Confirm Markings: Ensure manufacturer's identity, grade, cast/heat number are present and traceable.

- Check Packing List: Verify quantity, weight, dimensions against PO and MTC

- Quantity/Weight Check: Confirm total coil count and weight.

- Receiving of Coils in the system after verification

- After the incoming coil inspection, enter all the details of the coil in the Google sheet / ERP system of JSw One.

- The following data needs to be stored in the Google sheet / ERP system of Jsw One.

  - Mother coil number/batch id mentioned in the receiving invoice.

  - Coil number/batch id provided by CM for each coil received.

  - Input width

  - Input thickness

  - Grade

  - Weight

  - Inner Dia of Input coil

  - Quality Remarks: Ok / Not Ok / Hold

  - Remarks

- Surface Inspection: Visually inspect for presence of any surface defects such as seams, folds, cracks, pits, excessive scaling, waviness, kinks, twist, distortion in each incoming coils

- Dimensional Checks:  Measure thickness and width of each incoming coil.

- Weight Check: Measure weight of each incoming coil.

- Non-Conformance handling for incoming coils:

- In case of absence of identification and traceability of any of the incoming coil, hold the coil for processing and inform Contract Manufacturer and Coil Supplier. After discussion with Contract Manufacturer and Supplier, chemistry and dimension of the concerned coil(s) to be verified against the PO.

- In case of any mismatch of actual grade and size against PO, entire batch to be rejected for intended use or to be diverted for alternative use after consultation with relevant Contract Manufacturer and Coil supplier.

- In case of presence of surface defects, coils to be segregated and rejected.

- In case of thickness and width beyond acceptable range as specified in QAP/product standard, coils to be segregated and rejected.

- All rejected coils must be identified with RED TAG or RED PAINT and placed at a designated rejection area.

- Records & Reporting

- Records: Incoming Coil Joint Inspection Record

- Feedback: Provide feedback to suppliers, Contact Manufacturer and JSW One Team for evidence-based decision making and continual improvement.

| Created by: | Approved by: |
| --- | --- |
| JSW One Operational Head | JSW One |