|  | STANDARD OPERATING PROCEDURE | Document ID:JODL/PB PT/15 |
| --- | --- | --- |
|  |  | Version: 1 |
|  | Identification and Traceability of Pipes & Tubes | Date: 01/01/2026 |

Purpose:

To maintain unique identification and traceability of Pipes & Tubes from incoming stage to final product stage

Scope:

Intermediate and final product stage

Teams and Responsibilities:

| Sl No | Team | Responsibility |
| --- | --- | --- |
| 1 | Contract Manufacturer | Maintain stage wise identification and traceability |
| 2 | SGS QC Engineers | Ensure presence of proper identification and traceability during production and inspection activities |
| 3 | JSW One Operation Manager | Verification of identification and traceability in entire process route |

- Standard Operating Process:

- Verify presence of heat number and batch/coil number in each of the incoming coil.

- Assign a new coil number for each of the incoming coil in the below format:

OH <Location Name in 3 letters>YYMMDD-Sr. No.

E.g. 1st Coil of a particular lot received at Hyderabad Plant on 1st February, 2026 shall be assigned an internal coil number – OHHYD260201-1

- The original heat number and coil number of the each incoming coil shall be written in an incoming register as well as incoming coil inspection report against the newly assigned coil number to maintain the traceability.

- The slit coil or baby coil produced from a particular coil shall be assigned the original coil nomenclature followed by A, B, C, D

E.g. If four baby coils of same or multiple sizes are slit from one parent coil with a coil number OHHYD260201-1, the baby coil numbers shall be OHHYD260102-1A, OHHYD260102-1B, OHHYD260102-1C, OHHYD260102-1D.

5. Baby coil number shall be written in the slit register and slit inspection report

along with the respective dimensions such as width, thickness and length to maintain the traceability.

- The pipes produced from a specific baby coil, shall be assigned the same nomenclature as batch number.

E.g. All pipes produced from baby coil number OHHYD260102-1B shall be assigned a batch number OHHYD260102-1B.

7. Testing and inspection shall be performed and reported against this batch

number.

- Pipes under same batch number shall be packed in 1MT bundle and each of the bundle shall be assigned a bundle number. Bundle number shall be serial number of the bundle-01, 02, 03, 04……

- Two labels shall be attached with each bundle that contain batch number and bundle number along with dimensional details.

E.g. Labels of the first bundle from a particular batch, shall contain batch number OHHYD260102-1B and bundle number 01.

The combination of batch number and bundle number along with dimensional details shall form a unique identity for a particular bundle.

- In the production report batch number and bundle number wise record shall be maintained for traceability.

- In the Material Test Certificate (MTC), batch number shall be mentioned.

- Heat number and coil number of the incoming coil against the particular batch number shall be traced back from the incoming coil record.

- Records:

- Incoming coil inspection record

- Incoming coil record

- Joint inspection report of slit coil

- Joint inspection report of final pipes and tubes

- Production reports in slit and pipe stage

- Material Test Certificate

- Labels

| JSW One Quality Head | Contract Manufacturer |
| --- | --- |