|  | STANDARD OPERATING PROCEDURE | Document ID:JODL/PB PT/12 |
| --- | --- | --- |
|  |  | Version: 1.0 |
|  | Pre-Dispatch Inspection procedure for Pipes and Tubes | Date: 14/01/2026 |

Purpose:

To dispatch correct material as per Delivery Order and JSW One Quality Standards

Scope:

‘Ready to dispatch’ pipes and tubes product

Teams and Responsibilities:

| Sl No | Team | Responsibility |
| --- | --- | --- |
| 1 | Contract Manufacturer Loading and Dispatch Team | To ensure loading of correct materials in each vehicle as per delivery order/packing list |
| 2 | JSW One Operation & Dispatch team | To ensure dispatch of correct material as per delivery order in properly packed condition to the customer |

Standard Operating Process:

- JSW One Dispatch Executive shall be present during all vehicle loadings and deliveries

- Verify the numbers of bundles per size, type and grade of tube being loaded match with delivery order.

- Check size of minimum one tube per bundle to confirm the size

- Check whether any short length pipe is present in the bundles

- Check presence of One Helix Logo and other details on each pipe

- Ensure no pipe from other brand is present in the same vehicle

- Check labels containing all necessary information are present in each of the bundle

- In case of any mismatch or deviation in point no. 2 to 7, immediately hold the bundle and inform JSW and Contract Manufacturing team.

- Check each bundle is properly strapped and no loose bundle is being loaded

- Check the vehicle condition. If the vehicle condition is not appropriate or broken w.r.t dispatch SOP, inform JOTS team immediately.

- If the vehicle capacity is less than the quantity mentioned in the DO, hold the vehicle and inform JOTS team immediately

- If any surface/weld defect is detected at the time of dispatch, hold the lot and immediately inform JSW One and Contract Manufacturing QC Team.

- Fill and sign the Pre-Dispatch Inspection form

- If all the check points are found okay, clear the vehicle for dispatch.

- In case of any deviation/mismatch, hold the vehicle till the issue is fully resolved.

Record

Duly signed PDI report by JSW One Dispatch Executive

| JSW One Quality Head | Contract Manufacturer |
| --- | --- |