# One Helix Pipes & Tubes – Plant Operations SOP

Scope: This document provides Standard Operating Procedures (SOPs) for the complete manufacturing process of One Helix Pipes & Tubes, covering the workflow from HR Coil purchase and receipt through finished goods sales and yield calculation. This SOP is designed to guide the Plant Operations Team in tracking and managing the entire production lifecycle using the Pipes_and_Tubes-Operations.xlsx tracking system.

| Responsibilities | Maker | Checker |
| --- | --- | --- |
| HR Coil inspection | SGS Person | Sanjay Jha |
| HR Coil Tracking in master sheet | SGS Person | Sanjay Jha |
| Baby Coil Tracking | SGS Person | Sanjay Jha |
| FG Tracking | SGS Person | Sanjay Jha |
| FG Sales data Tracking | SGS Person | Sanjay Jha |

Document Sections:

A.1 - HR Coil Purchase & Receipt

A.2 - HR Coil Tracking

A.3 - HR Coil Slitting & Baby Coil Tracking

A.4 - FG Conversion & Tracking

A.5 - FG Sales/Dispatch Data

A.6 - Yield & Loss Calculation

# A.1 HR Coil Purchase & Receipt

This section describes the process for receiving and recording Hot Rolled (HR) coils procured from approved suppliers at the One Helix Pipes & Tubes manufacturing facility. Upon receipt, all coil details are recorded in the HRC PO sheet, and data will flow from this sheet to the coil sheet.

| Activity | Steps | Team | Interface / Utilities Used |
| --- | --- | --- | --- |
| 1. HR Coil Purchase & Receipt | 1. Receive HR coil delivery from approved supplier at plant facility 2. Verify physical condition of coil 3. Match delivery challan/invoice with purchase order 4. Confirm coil specifications against order details 5. Accept coil into inventory 6. Proceed to coil tracking and data entry | Plant Operations Team/SGS Team | Pipes_and_Tubes-Operations.xlsx Sheet: "HRC PO Sheet" |

Key Process Notes:

HR coils must be physically inspected for damage before acceptance

Delivery documentation must be verified against the purchase order

Only coils meeting quality specifications should be accepted

Upon acceptance, the coil proceeds to the tracking process (Section A.2)

# A.2 HR Coil Tracking

This section describes the process for recording detailed information of each HR coil in the Master Sheet (Coil sheet). All data entered in this step forms the master record. For tracking purposes, the plant team selects the HR Coil ID from a dropdown list linked to the Master Sheet, and the coil details are automatically populated in the HR Coil Tracking Sheet to ensure traceability throughout the manufacturing process.

| Activity | Steps | Team | Interface / Utilities Used |
| --- | --- | --- | --- |
| 2. HR Coil Data Entry & Tracking | 1. Open Pipes_and_Tubes-Operations.xlsx file 2. Navigate to "Coil Sheet" tab 3. Generate HR Coil ID using format: OH-<State Name - 2 Letters>-YYYYMMDD-Sequence number Example: OH-HR-20250115-01 4. Enter the Input Coil Number (Batch ID) from supplier documentation 5. Enter Coil Grade (e.g., JSW-SPHC, JSW-SPHD) 6. Enter Heat Number from coil identification markings 7. Enter Thickness (in mm) from specifications 8. Enter Length (in mm) from specifications 9. Enter Width (in mm) from specifications 10. Enter Weight (in kg) from weighbridge or documentation 11. Enter Cost Price per coil 12. Enter Purchase Order (PO) Number 13. Enter Date of Inward (receiving date) 14. Verify all entries for accuracy 15. Save the file | Plant Operations Team | Pipes_and_Tubes-Operations.xlsx Sheet: "Coil sheet" |

Coil ID Generation Logic:

Format: OH-<State- 2 digits>-YYYYMMDD-##

OH: One Helix prefix

State: Two-letter state code (e.g., HR for Haryana, UP for Uttar Pradesh)

YYYYMMDD: Date of receipt (Year-Month-Day)

##: Sequential number starting from 01 for each day

Example: OH-HR-20250115-01 (First coil received on January 15, 2025, in Haryana)

Critical Notes:

The HR Coil ID must be unique and follow the standardized format

All dimensional and weight data must be verified against supplier documentation

This master data will auto-populate in subsequent slitting and conversion processes

Ensure data accuracy as errors will cascade through the entire production chain

# A.3 HR Coil Slitting & Baby Coil Tracking

This section describes the process of slitting HR coils into smaller width coils (called "Baby Coils") and tracking each baby coil in the system. This is a critical step where one HR coil is divided into multiple baby coils for further processing.

| Activity | Steps | Team | Interface / Utilities Used |
| --- | --- | --- | --- |
| 3. HR Coil Slitting & Baby Coil Data Entry | 1. Open Pipes_and_Tubes-Operations.xlsx file 2. Navigate to "Coil to Slit" tab 3. Enter Date of Conversion (slitting date) 4. Enter Thickness and Width to be slit. Based on the entered parameters, the relevant HR Coil ID appears in a filtered dropdown. Select the applicable HR Coil ID 5. Generate Baby Coil Entries as A, B, C, D... for each slit section • Enter A for first baby coil • Enter B for second baby coil • Continue sequentially (C, D, E...) 6. Baby Coil ID auto-generates by concatenating: HR Coil ID + "-" + Baby Coil Entry Example: OH-HR-20250115-01-A 7. Thickness and Length auto-populate from master coil data 8. Enter Width (in mm) for each baby coil after slitting 9. Weight auto-calculates using formula: Weight = Thickness × Length × Width × Material Density 10. Cost Price, PO Number, and Date of Inward auto-populate from master data 11. Verify all entries for accuracy 12. Repeat steps 5-11 for each baby coil from the same HR coil 13. Save the file | Plant Operations Team | Pipes_and_Tubes-Operations.xlsx Sheet: "Coil to Slit" |

Baby Coil ID Generation Logic:

Format: [HR Coil ID]- [Baby Coil Entry]

Example: OH-HR-20250115-01-A

Critical Process Notes:

Each HR coil typically produces multiple baby coils depending on slitting requirements

Baby coil entries must be sequential (A, B, C...) for proper tracking

The sum of all baby coil widths should approximately equal the original HR coil width (accounting for wastage)

Weight calculations are automatic but should be verified against actual weighbridge measurements

Baby Coil IDs created here will be used in the FG Conversion process (Section A.4)

# A.4 FG Conversion & Tracking

This section describes the process of converting baby coils into finished goods (FG) bundles. Each baby coil is processed into multiple bundles based on SKU specifications, with each bundle representing a saleable unit of pipes/tubes.

| Activity | Steps | Team | Interface / Utilities Used |
| --- | --- | --- | --- |
| 4. FG Conversion & Bundle Creation | 1. Open Pipes_and_Tubes-Operations.xlsx file 2. Navigate to "FG Conversion" tab 3. Enter Date of Conversion (FG production date) 4. Select SKU Code from dropdown menu • SKU codes are pre-defined based on product specifications • Example: SKU-P25-100 (Pipe, 25mm thickness, 100mm width) 5. Baby Coil ID is auto-populated from the "Coil to Slit" sheet based on the selected SKU Code 6. Generate Bundle ID Entry as 01, 02, 03... for each bundle • Enter 01 for first bundle from this baby coil • Enter 02 for second bundle • Continue sequentially (03, 04, 05...) 7. Bundle ID (Unique ID) auto-generates by concatenating: Baby Coil ID + "-" + Bundle Entry Example: OH-HR-20250115-01-A-01 8. Enter Number of Pieces per bundle 9. Enter Length (in mm) of each piece 10. Width and Thickness auto-populate from baby coil data 11. Enter Weight (in kg) of the completed bundle 12. Verify all entries for accuracy 13. Repeat steps 6-12 for each bundle from the same baby coil 14. Save the file | Plant Operations Team | Pipes_and_Tubes-Operations.xlsx Sheet: "FG Conversion" |

Bundle ID Generation Logic:

Format: [Baby Coil ID]-[Bundle Entry Number]

Example: OH-HR-20250115-01-A-01

Critical Process Notes:

Each baby coil can produce multiple bundles depending on production requirements

Bundle entries must be sequential (01, 02, 03...) for proper tracking

SKU Code selection determines the final product specifications

Bundle IDs created here will be used in the FG Sales/Dispatch process (Section A.5)

Weight should be verified against theoretical weight for yield calculation purposes

Each bundle represents a complete, saleable unit ready for dispatch

# A.5 FG Sales/Dispatch Data

This section describes the process of recording finished goods sales and dispatch information. This is the final stage before products leave the facility, where actual dispatch weights are captured and compared against theoretical weights to calculate yield.

| Activity | Steps | Team | Interface / Utilities Used |
| --- | --- | --- | --- |
| 5. FG Sales/Dispatch Recording | 1. Open Pipes_and_Tubes-Operations.xlsx file 2. Navigate to "FG sales" tab 3. Select SKU Code from dropdown menu • Must match the SKU from FG Conversion process 4. Select Baby Coil ID from dropdown (linked to "FG Conversion" sheet) 5. Select Bundle ID from dropdown (linked to "FG Conversion" sheet) • Only bundles matching the selected Baby Coil ID will appear 6. Enter Number of Pieces being dispatched. • Can be full bundle or partial dispatch 7. Thickness, Length, Width auto-populate from FG Conversion data 8. Theoretical Weight auto-calculates based on: Theoretical Weight = Number of Pieces × Length × Width × Thickness × Material Density 9. Enter Actual Weight from weighbridge at dispatch • This is the actual measured weight of the dispatched goods 10. Variation auto-calculates as: Variation = Actual Weight - Theoretical Weight • Negative value indicates weight loss/scrap • Positive value indicates weight gain (rare, may indicate measurement error) 11. Verify all entries for accuracy 12. Save the file 13. Generate dispatch documentation (invoice, delivery challan) 14. Release goods for dispatch | Plant Operations Team | Pipes_and_Tubes-Operations.xlsx Sheet: "FG sales" |

Variation Analysis:

Negative Variation (Red): Indicates weight loss due to:

Processing scrap

Cutting waste

Edge trimming

Material defects removed

Measurement differences

Critical Process Notes:

Always verify actual weight using calibrated weighbridge before dispatch

Variation data feeds directly into Yield Calculation (Section A.6)

Consistent negative variations may indicate process inefficiencies requiring investigation

Large variations (>5% of theoretical weight) should be flagged for quality review

This data provides complete traceability from raw material (HR Coil) to finished product dispatch

# A.6 Yield & Loss Calculation

This section describes the methodology for calculating manufacturing yield based on the variation between theoretical weight and actual weight recorded during the FG sales/dispatch process. Yield calculation is critical for measuring process efficiency, overall conversion efficiency, and identifying areas for improvement. The variance analysis enables better production control and cost analysis.

| Activity | Steps | Team | Interface / Utilities Used |
| --- | --- | --- | --- |
| 6. Yield & Loss Calculation & Analysis | 1. Open Pipes_and_Tubes-Operations.xlsx file 2. Navigate to "FG sales" tab 3. Review Variation column data for all dispatched bundles 4. Calculate Individual Bundle Yield: Bundle Yield (%) = (Actual Weight / Theoretical Weight) × 100 5. Calculate Aggregate Yield for reporting period: Period Yield (%) = (Total Actual Weight / Total Theoretical Weight) × 100 6. Analyze yield by: • HR Coil batch • Baby Coil • SKU Code • Production date/period • Operator/shift 7. Identify patterns in yield loss 8. Calculate material loss: Material Loss (kg) = Total Theoretical Weight - Total Actual Weight Material Loss (%) = (Material Loss / Total Theoretical Weight) × 100 9. Document findings in yield analysis report 10. Recommend corrective actions for: • Yield < 95%: Minor process optimization needed • Yield < 90%: Significant process investigation required • Yield < 85%: Immediate corrective action required 11. Track improvement over time 12. Update process documentation based on findings | Planning team | Pipes_and_Tubes-Operations.xlsx All sheets for comprehensive analysis |

Yield Calculation Formulas:

| Metric | Formula |
| --- | --- |
| Bundle Yield | Yield (%) = (Actual Weight / Theoretical Weight) × 100 |
| Material Loss (Weight) | Loss (kg) = Theoretical Weight - Actual Weight |
| Material Loss (Percentage) | Loss (%) = (Material Loss / Theoretical Weight) × 100 |

Yield Benchmarks & Action Triggers:

| Yield Range | Status | Action Required |
| --- | --- | --- |
| ≥ 98% | Excellent | No action required, maintain current process |
| 95% - 97.9% | Good | Monitor for trends, minor optimization opportunities |
| 90% - 94.9% | Moderate | Process investigation required, identify root causes |
| < 90% | Poor | Immediate corrective action, halt production if necessary |

Key Performance Indicators (KPIs):

Overall Manufacturing Yield: Target ≥ 96%

Slitting Yield (HR Coil to Baby Coil): Target ≥ 98%

Conversion Yield (Baby Coil to FG): Target ≥ 95%

Material Loss Rate: Target ≤ 4%

Scrap Generation: Target ≤ 3% of input weight

Common Causes of Yield Loss:

Edge trimming and width adjustments during slitting (1-2% typical)

Length cutting waste (0.5-1% typical)

Quality defects requiring removal (0.5-1.5% typical)

Surface conditioning and finishing (0.3-0.8% typical)

Measurement and weighing variations (0.1-0.3% typical)

Material handling and storage losses (0.1-0.2% typical)

Yield Improvement Actions:

Optimize slitting parameters to minimize edge waste

Implement length optimization software for cutting schedules

Enhance quality control at HR coil receipt stage

Regular calibration of weighbridge and measuring equipment

Operator training on material handling best practices

Review and update SKU specifications to match coil dimensions

Critical Notes:

Yield calculations must be performed daily for immediate visibility into process performance

Monthly yield reports should include trend analysis and corrective action tracking

Yield data should be reviewed in production meetings to drive continuous improvement

Significant yield deviations (>5% from target) require root cause analysis within 24 hours

Yield improvement initiatives should be tracked with measurable targets and timelines