## Process Flow: HR Coil Procurement to Finished Goods (FG) Dispatch

### 1. HR Coil Purchase & Receipt

HR coils are procured from approved suppliers and received at the plant. Once received, all coil details are recorded in the Master Sheet.

Details captured include:

- HR Coil ID

- Heat Number

- Coil Grade

- Thickness

- Length

- Width

- Weight

- Cost Price

- Purchase Order (PO) Number

- Date of Inward

### 2. HR Coil Tracking

- The plant team selects the received HR Coil ID from a dropdown list linked to the Master Sheet.

- Selected coil details are automatically populated in the HR Coil Tracking Sheet to ensure traceability.

### 3. HR Coil Slitting & Baby Coil Tracking

- The plant team records the slitting (conversion) date.

- Thickness and width to be slit are entered.

- Based on the entered parameters, the relevant HR Coil ID appears in a dropdown, from which the team selects the applicable coil.

- Baby coils are labeled sequentially as A, B, C, etc.

- Baby Coil ID is auto-generated by concatenating the HR Coil ID + Baby Coil letter.

- Thickness and length are auto-populated from the HR Coil data.

- Width is entered manually by the plant team.

- Weight is calculated automatically using a predefined formula.

- Cost price, PO number, and inward date are captured for each baby coil to maintain cost traceability.

### 4. Finished Goods (FG) Conversion & Tracking

- The plant team enters the FG conversion date.

- SKU Code is selected from a predefined dropdown list.

- Baby Coil ID is auto-populated.

- Bundles are numbered sequentially as 01, 02, 03, etc.

- Bundle ID is auto-generated by concatenating the Baby Coil ID + Bundle number.

- The plant team enters:

  - Number of pieces

  - Length

  - Weight

- Thickness and width are auto-populated from the baby coil data.

### 5. FG Sales / Dispatch Data

For dispatch of finished goods, the plant team records:

- SKU Code

- Baby Coil ID (selected from dropdown)

- Bundle ID (selected from dropdown)

- Number of pieces

- Thickness, length, width, and theoretical weight are auto-calculated.

- The plant team then enters the actual FG weight at the time of dispatch.

### 6. Yield & Loss Calculation

- Yield is calculated by comparing the theoretical weight with the actual dispatched weight.

- The variance highlights process losses, scrap generation, and overall conversion efficiency, enabling better production control and cost analysis.